Limited Sportsman/ Sport Series / Super Street Open Rules

1. Stock type Chassis / No tube type chassis allowed (may have tubing in front of radiator and behind rear axle). Stock floor pan in stock location must be retained. Cars utilizing aftermarket interior or firewalls are permitted with additional 25lbs total weight.

2. Shocks must be mounted in stock location. No heim end or coil over type shocks permitted. No external adjustable shocks permitted.

3. Wheels: 7” or 8” steel wheels. Maximum 2” wheel offset

4. Tires: American Racer 7” or 8” 704,705,or706 series only. No tire softening permitted!

5. Spoiler: 6” High by 60” wide maximum. Maximum height measured from ground surface shall be 41”, measured at highest point. One spoiler per car mounted at the rear of the deck lid or tail section. No roof vanes or roof spoilers permitted.

6. Carburetion: Stock factory 2-barrel carburetor or Holley 4412 2 barrel permitted. Holley must be stock with removal of choke plate and must pass tech tool.

7. Factory cast Iron intake or Edelbrock aluminum 2101 or 2116 only. Maximum 1” carburetor adapter/spacer allowed.

8. Flat top pistons only.

9. Weight: 2900lbs. With driver before race. 56% left side

10. Rear end: Factory type only. NO quick change rears permitted

11. Steel body only. Aftermarket nose and tails permitted. Rear tail(bumper cover) area must be enclosed. No extended tails permitted! Cars utilizing aftermarket body panels are permitted with an additional 50lbs to total weight. Must be neat appearing and appearance must be appropriate to stock type desired configuration.

12. Minimum 3” ground/frame clearance

13. 4 wheel working brakes mandatory.

14. Added weight must securely fastened and painted white.

15. Full windshield required.

16. Normal standard safety rules apply.

These rules are intended for fair and equitable competition. This race is designed for the division of cars listed at the heading. Management has the right to deem any car not within the rightful division