STOCK 8  OPEN RULES

- Minimum wheel base 101 inches

- American made passenger car  models only. No leaf spring type chassis permitted( I.E. Camaro-Firebird)

- Only two wheel drive models allowed.

- All glass except windshield must be removed. windshield  may be replaced with lexan type material. No rear or side windows permitted.

- Exhaust must extend behind driver and dump under the car.

- All doors must be welded, bolted or chained shut. Hood and rear deck lid must have pull pin type fasteners.

- Stock front and rear bumpers for year, make and model must remain in stock location. No aftermarket bumper covers allowed.

- All body panels must remain stock for year, make and model. No cutting of panels except for drivers door for roll bar(door bar) clearance.

- All upholstery, additional seats, and any flammable articles must be removed.

- Original stock floor boards and firewalls must remain in place, any holes must be covered with minimum 22 gauge steel.

- Battery must remain under hood. Must securely mounted and covered.

- Minimum 4 point roll cage is mandatory. Hatchback type chassis may not have tubing extended beyond the center of the rear axle. Trunk area must be enclosed and sealed from drivers compartment.

- Quick release window net is mandatory.

- Fire extinguisher securely mounted within drivers reach is mandatory.

- Quick release steering wheel permitted.

- Springs must stock for year, make and model. Springs must be identical wire size, number of coils and same free height from left to right.

- Frame height: minimum frame height will be 6 inches measured from the lowest point of the frame rail and measured without driver.

- Rear ends shall be stock for year, make and model. To include gear ratio.

- Fuel cells or boat tanks permitted. Must be centered in trunk and securely mounted above lowest point of rear frame.

- 5 point harness safety harnesses mandatory.  Drivers suit highly recommended.

- TIRES: must be DOT type radial, same size, same number,15 inch 65, 70 or 75 series only.16 inch may use 60 series with approved tire. Maximum 7 inch wheel width permitted. Maximum wheel offset not to exceed 2 inch as measured from back of wheel lip. . One inch lugs permitted.

- Camber:  ONE inch maximum.

- Transmission: stock  for year, make and model.

- ENGINE: 6 or 8 cylinder allowed. Engine must be stock for year, make and model. All parts of engine must remain stock to year, make and model used.

- Anything not mentioned within these rules shall remain stock for year, make and model used unless otherwise approved by tech……..

ELIGIBLE COMPETEING DIVISIONS

OUTLAW-MAD BOMBER-STOCK 8

ANY COMPETITORS ARE ELIGIBLE BUT MUST ADHERE TO ABOVE MENTIONED RULE GUIDELINES